- A429信号数据位定义

### 1  起落架系统接收信号位定义

#### 1.1  Label TBD UTC Time（UTC时间）

| Label | TBD |  |  |
| --- | --- | --- | --- |
| Name | UTC Time |  |  |
| Description | Clock; Received by LGCU Control Channel |  |  |
| BIT | FUNCTION | BIT STATUS |  |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |  |
| 9-10 | SDI | 10,9 Description 00: default 01: not used 10: not used 11: Not used |  |
| 11(LSB) | GNSS System Clock used as source | 0=not GNSS System Clock | 1=GNSS System Clock |
| 12-17 (LSB-MSB) | Seconds: 12 = 1; 13 = 2; 14 = 4; 15 = 8; 16 = 16; 17 = 32; |  |  |
| 18-23 (LSB-MSB) | Minutes: 18 = 1; 19 = 2; 20 = 4; 21 = 8; 22 = 16; 23 = 32; |  |  |
| 24-28 (LSB-MSB) | Hours: 24 = 1; 25 = 2; 26 = 4; 27 = 8; 28 = 16; |  |  |
| 29 | Spare (0) |  |  |
| 30, 31 | Status Matrix | 31,30 Description 00: Failure Warning 01: No Computed Data 10: Functional Test 11: Normal Operation |  |
| 32 | Parity – Odd |  |  |

#### 1.4  Label TBD Flight Leg（OHMS飞行航段和全局终止标志）

| Label | TBD |  |  |
| --- | --- | --- | --- |
| Name | Flight Leg |  |  |
| Description | Units: Count Range (scale): -128 to +127 Significant Bits: 7 Resolution: 1 Encoding Data Type: Signed 2’s Complement BNR Complement BNR OMS Flight Leg integer value, incrementing each time the flight leg changes; when the current flight leg is +127 and a new flight leg starts, the transmitted OMS Flight Leg value will start again at -128. The sign value is determined using bit 29. Note: |  |  |
| BIT | FUNCTION | BIT STATUS |  |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |  |
| 9-10 | SDI | 10,9 Description 00: default 01: Not used 10: Not used 11: Not used |  |
| 11 | OHMS全局终止标志布尔值 | 0=FALSE | 1=TRUE |
| 12-21 | Spare (0) |  |  |
| 22 | 1 |  |  |
| 23 | 2 |  |  |
| 24 | 4 |  |  |
| 25 | 8 |  |  |
| 26 | 16 |  |  |
| 27 | 32 |  |  |
| 28 | 64 |  |  |
| 29(MSB) | Sign | 0 = plus | 1 = minus |
| 30, 31 | Status Matrix | 31,30 Description 00: Failure Warning 01: No Computed Data 10: Functional Test 11: Normal Operation |  |
| 32 | Parity – Odd |  |  |

#### 1.5  Label TBD UTC Date （UTC日期）

| Label | TBD |  |
| --- | --- | --- |
| Name | UTC Date |  |
| Description | Date; Received by LGCU Control Channel |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: default 01: Not used 10: Not used 11: Not used |
| 11-14 (LSB-MSB) | Year (ones): 11=1; 12=2; 13=4; 14=8; |  |
| 15-18 (LSB-MSB) | Year (tens): 15=10; 16=20; 17=40; 18=80; |  |
| 19-22 (LSB-MSB) | Month (ones): 19=1; 20=2; 21=4; 22=8 |  |
| 23 | Month (tens): 23 = 10 |  |
| 24-27 (LSB-MSB) | Day (ones): 24=1; 25=2; 26=4; 27=8 |  |
| 28-29 (LSB-MSB) | Day (tens): 28=10; 29=20 |  |
| 30, 31 | Status Matrix | 31,30 Description 00: Plus 01: No Computed Data 10: Functional Test 11: Minus |
| 32 | Parity – Odd |  |

#### 1.6  Label TBD A/C Registration Number Part 1（飞机注册号）

| Label | 301 |  |
| --- | --- | --- |
| Name | A/C Registration Number Part 1 |  |
| Description | Encoding Data Type: ISO #5 Note: |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: default 01: Not used 10: Not used 11: Not used |
| 11-17 (LSB)-(MSB) | Character 1 |  |
| 18-24 (LSB)-(MSB) | Character 2 |  |
| 25-31 (LSB)-(MSB) | Character 3 |  |
| 32 | Parity – Odd |  |

#### 1.7  Label TBD A/C Registration Number Part 2（飞机注册号）

| Label | TBD |  |
| --- | --- | --- |
| Name | A/C Registration Number Part 2 |  |
| Description | Encoding Data Type: ISO #5 Note: |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: default 01: Not used 10: Not used 11: Not used |
| 11-17 (LSB)-(MSB) | Character 4 |  |
| 18-24 (LSB)-(MSB) | Character 5 |  |
| 25-31 (LSB)-(MSB) | Character 6 |  |
| 32 | Parity – Odd |  |

#### 1.8  Label TBD A/C Registration Number Part 3（飞机注册号）

| Label | 303 |  |
| --- | --- | --- |
| Name | A/C Registration Number Part 3 |  |
| Description | Encoding Data Type: ISO #5 Note: |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: default 01: Not used 10: Not used 11: Not used |
| 11-17 (LSB)-(MSB) | Character 7 |  |
| 18-24 (LSB)-(MSB) | Character 8 |  |
| 25-31 (LSB)-(MSB) | Character 9 |  |
| 32 | Parity – Odd |  |

### 2  起落架系统发送信号位定义

#### 2.1  Label TBD LG Prox LGCU1 Monitor Channel

| Label | 144（八进制） |  |
| --- | --- | --- |
| Name | LG Prox LGCU1 Monitor Channel |  |
| Description | Landing Gear Sensor evaluated by the Monitor Channel of LGCU1 |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_MonCh_Tx1 10: Not used 11: Not used |
| 11 | RH_MLG_Prox_WOFFW_1 | 0=Not WOFFW 1=WOFFW |
| 12 | RH_MLG_Prox_WOFFW_1_OP | 0=Not Operational 1=Operational |
| 13-14 | Spare |  |
| 15 | RH_MLG_Prox_DNLK_1 | 0=Not Downlocked 1=Downlocked |
| 16 | RH_MLG_Prox_DNLK_1_OP | 0=Not Operational 1=Operational |
| 17-21 | Spare |  |
| 22 | Spare |  |
| 23-29 | Spare |  |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not Used 10: Not Used 11: Failure Warning |
| 32 | Parity - Odd |  |

#### 2.2  Label TBD LG Prox Monitor Channel LGCU2

| Label | 145（八进制） |  |
| --- | --- | --- |
| Name | LG Prox Monitor Channel LGCU2 |  |
| Description | Landing Gear Sensor evaluated by the Monitor Channel of LGCU2 |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: Not used 10: A429_LGCU2_MonCh_Tx1 11: Not used |
| 11 | LH_MLG_Prox_WOFFW_2 | 0=Not WOFFW 1=WOFFW |
| 12 | LH_MLG_Prox_WOFFW_2_OP | 0=Not Operational 1=Operational |
| 13-14 | Spare |  |
| 15 | LH_MLG_Prox_DNLK_2 | 0=Not Downlocked 1=Downlocked |
| 16 | LH_MLG_Prox_DNLK_2_OP | 0=Not Operational 1=Operational |
| 17-21 | Spare |  |
| 22 | Spare |  |
| 23-29 | Spare |  |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not Used 10: Not Used 11: Failure Warning |
| 32 | Parity - Odd |  |

#### 2.3  Label TBD LGCU Configuration Data

| Label | 146（八进制） |  |
| --- | --- | --- |
| Name | LGCU Configuration Data |  |
| Description | Configuration Data (PNR, SNR, CRC automatic mode Tx) Transmitted by LGCU Control Channel |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-31 | Per ARINC protocol – automatic mode |  |
| 32 | Parity - Odd |  |

#### 2.4  Label TBD LG Position Sensors

| Label | 147（八进制） |  |
| --- | --- | --- |
| Name | LG Position Sensors |  |
| Description | Status of LG Position Indication Sensors; Transmitted by LGCU Control Channel |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | MLG RH Uplock Proxi Sensor | 0=not Uplocked 1=Uplocked |
| 12 | MLG LH Uplock Proxi Sensor | 0=not Uplocked 1=Uplocked |
| 13 | NLG Uplock Proxi Sensor | 0=not Uplocked 1=Uplocked |
| 14 | MLG RH Downlock Proxi Sensor | 0=not Downlocked 1=Downlocked |
| 15 | MLG LH Downlock Proxi Sensor | 0=not Downlocked 1=Downlocked |
| 16 | NLG Downlock Proxi Sensor | 0=not Downlocked 1=Downlocked |
| 17 | MLG RH Uplock Proxi Sensor operational | 0=not Operational 1=Operational |
| 18 | MLG LH Uplock Proxi Sensor operational | 0=not Operational 1=Operational |
| 19 | NLG Uplock Proxi Sensor operational | 0=not Operational 1=Operational |
| 20 | MLG RH Downlock Proxi Sensor operational | 0=not Operational 1=Operational |
| 21 | MLG LH Downlock Proxi Sensor operational | 0=not Operational 1=Operational |
| 22 | NLG Downlock Proxi Sensor operational | 0=not Operational 1=Operational |
| 23 | LGCL selected position up (retraction) | 0=not up 1=up |
| 24 | LGCL selected position down (extension) | 0=not down 1=down |
| 25 | LGCL selected position up operational | 0=not Operational 1=Operational |
| 26 | LGCL selected position down operational | 0=not Operational 1=Operational |
| 27 | Spare |  |
| 28 | Command Retract | 0=Not Command Retract 1=Command Retract |
| 29 | Command Extend | 0=Not Command Extend 1=Command Extend |
| 30-31 | SSM | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

#### 2.5  Label TBD Consolidated LG Position Data

| Label | 148（八进制） |  |
| --- | --- | --- |
| Name | Consolidated LG Position Data |  |
| Description | Consolidated Status of LG Position |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | All Gear Downlocked | 0=not Downlocked 1=Downlocked |
| 12 | All Gear Uplocked | 0=not Uplocked 1=Uplocked |
| 13 | NLG Uplocked | 0=not Uplocked 1=Uplocked |
| 14 | NLG Downlocked | 0=not Downlocked 1=Downlocked |
| 15 | Extension Driver energized | 0=not energized 1=energized |
| 16 | Retraction Driver energized | 0=not energized 1=energized |
| 17 | Spare |  |
| 18 | Spare |  |
| 19 | Spare |  |
| 20 | Spare |  |
| 21 | Spare |  |
| 22 | AES Relay 1 Driver Low energized (only LGCU2) | 0=not energized 1=energized |
| 23 | AES Relay 3-5 Driver Low energized (only LGCU2) | 0=not energized 1=energized |
| 24 | LgclUpDn (internal use) | 0 = False 1 = True |
| 25 | Spare |  |
| 26 | Spare |  |
| 27 | Consolidated MLG Not Downlocked | 0=Downlocked 1=not Downlocked |
| 28 | Consolidated MLG Uplocked | 0=not Uplocked 1=Uplocked |
| 29 | LgclDnUp (internal use) | 0 = False 1 = True |
| 30-31 | SSM | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

Note: For LGCU 1, Label 273 BIT 20-23 are defined as spare.

#### 2.6  Label TBD WOFFW Data

| Label | 149（八进制） |  |
| --- | --- | --- |
| Name | WOFFW Sensor Data |  |
| Description | WOFFW Sensor Status Data; Transmitted by LGCU Control Channel |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | MLG RH WOFFW Proxi Sensor | 0=not WOFFW 1=WOFFW |
| 12 | MLG LH WOFFW Proxi Sensor | 0=not WOFFW 1=WOFFW |
| 13 | NLG WOFFW Proxi Sensor | 0=not WOFFW 1=WOFFW |
| 14 | MLG RH WOFFW Proxi Sensor operational | 0=not Operational 1=Operational |
| 15 | MLG LH WOFFW Proxi Sensor operational | 0=not Operational 1=Operational |
| 16 | NLG WOFFW Proxi Sensor operational | 0=not Operational 1=Operational |
| 17 | Spare |  |
| 18 | Spare |  |
| 19 | Spare |  |
| 20 | Spare |  |
| 21 | Spare |  |
| 22-29 | Spare |  |
| 30-31 | SSM | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

#### 2.7  Label TBD Consolidated WOW Data

| Label | 150（八进制） |  |
| --- | --- | --- |
| Name | Consolidated WOW Data |  |
| Description | WOW Data; Transmitted by LGCU Control Channel |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | Spare |  |
| 12 | All Gear WOW | 0=not WOW 1=WOW |
| 13 | MLG WOW (LH and RH) | 0=not WOW 1=WOW |
| 14 | GR_ WOFFW | 0=not WOFFW 1=WOFFW |
| 15 | Spare |  |
| 16 | NLG Uplock Switch Off Monitor(LGCU1 only) | 0 = not in OFF position 1 = in OFF position |
| 17 | MLG RH Uplock Switch Off Monitor (LGCU2 only) | 0 = not in OFF position 1 = in OFF position |
| 18 | MLG LH Uplock Switch Off Monitor (LGCU2 only) | 0 = not in OFF position 1 = in OFF position |
| 19 | NLG Door Uplock Switch Off Monitor (LGCU1 only) | 0 = not in OFF position 1 = in OFF position |
| 20 | MLG RH Door Uplock Switch Off Monitor(LGCU1 only) | 0 = not in OFF position 1 = in OFF position |
| 21 | MLG LH Door Uplock Switch Off Monitor(LGCU1 only) | 0 = not in OFF position 1 = in OFF position |
| 22 | Spare |  |
| 23 | DI_MON_AES_GND_DISC | 0 = GND connected 1 = GND disconnected |
| 24 | DI_AES_28V_EN (LGCU2 only) | 0=28V not enabled 1=28V enabled |
| 25 | All Gear WOW operational | 0=not Operational 1=Operational |
| 26 | MLG WOW (LH and RH) operational | 0=not Operational 1=Operational |
| 27-29 | Spare |  |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

Note: For LGCU 1, Label 275 BIT 17-18，24 are defined as spare. For LGCU 2, Label TBD BIT 16, 19-21 are defined as spare.

#### 2.8  Label TBD LG Position Indication Status for CAS

| Label | 151（八进制） |  |
| --- | --- | --- |
| Name | LG Position Indication Status for CAS |  |
| Description | LG position and failure data; Transmitted by LGCU Control Channel |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | LG System non Dispatchable Fault | 0=no fault 1=fault |
| 12 | LGCU Internal Fault | 0=no fault 1=fault |
| 13 | LG System Degraded | 0=not degraded 1=degraded |
| 14 | WOW System Fault | 0=no fault 1=fault |
| 15 | WOW System Degraded | 0=not degraded 1=degraded |
| 16 | Alternate Extension LG down | 0=not down 1=down |
| 17 | Alternate Extension LG fault | 0=no fault 1=fault |
| 18 | NG Disagree (NLG) | 0=not disagree 1=disagree |
| 19 | LG Disagree (MLG LH) | 0=not disagree 1=disagree |
| 20 | RG Disagree (MLG RH) | 0=not disagree 1=disagree |
| 21 | NG Uplocked (NLG) | 0=not uplocked 1=uplocked |
| 22 | LG Uplocked (MLG LH) | 0=not uplocked 1=uplocked |
| 23 | RG Uplocked (MLG RH) | 0=not uplocked 1=uplocked |
| 24 | NG In Transit (NLG) | 0=not In Transit 1=In Transit |
| 25 | LG In Transit (MLG LH) | 0=not In Transit 1=In Transit |
| 26 | RG In Transit (MLG RH) | 0=not In Transit 1=In Transit |
| 27 | NG downlocked (NLG) | 0 = not downlocked 1 = downlocked |
| 28 | LG downlocked (MLG LH) | 0 = not downlocked 1 = downlocked |
| 29 | RG downlocked (MLG RH) | 0 = not downlocked 1 = downlocked |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

#### 2.9  Label TBD LGCU 1/2 Maintenance Word 1 (LGCIS / AES faults)

| Label | 152（八进制） |  |
| --- | --- | --- |
| Name | LGCU 1/2 Maintenance Word 1 (LGCIS / AES faults) |  |
| Description | LGS failure occurrence for trouble shooting; Transmitted by LGCU Control Channel |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | LGCU Non Dispatchable Fault (only LGCU1 ) | 0=no fault 1=fault |
| 12 | AES Relay 3 Non Dispatchable Fault (only LGCU2) | 0=no fault 1=fault |
| 13 | AES Relay 3 Electrical Fault (only LGCU2) | 0=no fault 1=fault |
| 14 | AES Relay 2 Non Dispatchable Fault (only LGCU2) | 0=no fault 1=fault |
| 15 | AES Relay 1 electrical Fault (only LGCU2) | 0=no fault 1=fault |
| 16 | AES Relay 2 electrical Fault (only LGCU2) | 0=no fault 1=fault |
| 17 | AES NG Uplk electrical Fault (only LGCU1) | 0=no fault 1=fault |
| 18 | AES LG Uplk electrical Fault (only LGCU2) | 0=no fault 1=fault |
| 19 | AES RG Uplk electrical Fault (only LGCU2) | 0=no fault 1=fault |
| 20 | Spare |  |
| 21 | Spare |  |
| 22 | EHM3 Fault | 0=no fault 1=fault |
| 23 | LGCL electrical Fault | 0=no fault 1=fault |
| 24 | LGCL non Dispatchable Fault | 0=no fault 1=fault |
| 25 | AEV Position2 electrical Fault(only LGCU2) | 0=no fault 1=fault |
| 26 | EHM1 Fault | 0=no fault 1=fault |
| 27 | EHM2 Fault | 0=no fault 1=fault |
| 28 | AEV Position1 electrical Fault | 0=no fault 1=fault |
| 29 | AES Relay 1 Non Dispatchable Fault (only LGCU2) | 0=no fault 1=fault |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

#### 2.10  Label TBD LGCU 1/2 Maintenance Word 2 (Adj Requ of LG Prox)

| Label | 153（八进制） |  |
| --- | --- | --- |
| Name | LGCU 1/2 Maintenance Word 2 (Adj Requ of LG Prox) |  |
| Description | LGS Sensor Adjustment required information; Each Sensor 1 transmitted by LGCU1 CC and Sensor 2 by LGCU2 CC |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | MLG LH WOFFW Proxi Sensor adjustment | 0 = adjust. not required 1 = adjust required |
| 12 | MLG RH WOFFW Proxi Sensor adjustment | 0 = adjust. not required 1 = adjust required |
| 13 | NLG WOFFW Proxi Sensor adjustment | 0 = adjust. not required 1 = adjust required |
| 14 | MLG LH downlock Proxi Sensor adjustment | 0 = adjust. not required 1 = adjust required |
| 15 | MLG RH dowlock Proxi Sensor adjustment | 0 = adjust. not required 1 = adjust required |
| 16 | NLG downlock Proxi Sensor adjustment | 0 = adjust. not required 1 = adjust required |
| 17 | NLG uplock Proxi Sensor adjustment | 0 = adjust. not required 1 = adjust required |
| 18 | MLG RH uplock Proxi Sensor adjustment | 0 = adjust. not required 1 = adjust required |
| 19 | MLG LH uplock Proxi Sensor adjustment | 0 = adjust. not required 1 = adjust required |
| 20 | Spare |  |
| 21 | Spare |  |
| 22 | Spare |  |
| 23 | Spare |  |
| 24 | Spare |  |
| 25 | Spare |  |
| 26 | Spare |  |
| 27 | Spare |  |
| 28 | Spare |  |
| 29 | Spare |  |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

#### 2.11  Label TBD LGCU 1/2 Maintenance Word 5

| Label | 154（八进制） |  |
| --- | --- | --- |
| Name | LGCU 1/2 Maintenance Word 5 |  |
| Description | Transmitted by LGCU CC |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | Spare |  |
| 12 | Spare |  |
| 13 | AES Relay 4 Non Dispatchable Fault (only LGCU2) | 0=not fault 1=fault |
| 14 | AES Relay 4 Electrical Fault (only LGCU2) | 0=not fault 1=fault |
| 15 | AES Relay 5 Non Dispatchable Fault (only LGCU2) | 0=not fault 1=fault |
| 16 | AES Relay 5 Electrical Fault (only LGCU2) | 0=not fault 1=fault |
| 17 | Spare |  |
| 18-29 | Spare |  |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

Note: For Label 354:

Bit 13 AES Relay 4 Non Dispatchable Fault: LGCU2 Only

Bit 14 AES Relay 4 Electrical Fault: LGCU2 Only

Bit 15 AES Relay 5 Non Dispatchable Fault: LGCU2 Only

Bit 16 AES Relay 5 Electrical Fault: LGCU2 Only

#### 2.12  Label TBD LGCU 1/2 Maintenance Word 6 (Invalid ARINC Data)

| Label | 155（八进制） |  |
| --- | --- | --- |
| Name | LGCU 1/2 Maintenance Word 6 (Invalid ARINC Data) |  |
| Description | ARINC Availability information Transmitted by LGCU CC |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | Invalid data of Time, Date, Flight Leg, A/C Registration | 0=valid data 1=no valid data |
| 12 | Spare |  |
| 13 | Spare |  |
| 14 | Spare |  |
| 15 | Spare |  |
| 16 | Spare |  |
| 17 | Spare |  |
| 18 | Spare |  |
| 19 | Spare |  |
| 20 | Spare |  |
| 21 | Spare |  |
| 22 | Spare | Spare |
| 23 | Spare | Spare |
| 24 | Spare | Spare |
| 25 | Invalid Crosscom Control Channel | 0=valid data 1=no valid data |
| 26 | Spare |  |
| 27 | Spare | Spare |
| 28-29 | Spare |  |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

Note: For Label 355:

Bit 13 Invalid FCM 1 Rudder Pedal data for ConCh Rx2: LGCU1 Only

Bit 14 Invalid FCM 2 Rudder Pedal data for ConCh Rx3: LGCU1 Only

Bit 16 Invalid FCM 3 Rudder Pedal data for MonCh Rx2: LGCU1 Only

Bit 17 Invalid FCM 2 Rudder Pedal data for MonCh Rx3: LGCU1 Only

Bit 27 Pedal Command Asymmetry Monitor Fault: LGCU1 Only

#### 2.13  Label TBD LGCU 1/2 Maintenance Word 8 (LG Sensor Unreasonable)

| Label | 156（八进制） |  |
| --- | --- | --- |
| Name | LGCU 1/2 Maintenance Word 8 (LG Sensor Unreasonable) |  |
| Description | LG sensor unreasonable information; Transmitted by LGCU CC |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | MLG LH WOFFW Proxi unreasonable | 0 = not unreason. 1 = unreason. |
| 12 | MLG RH WOFFW Proxi unreasonable | 0 = not unreason. 1 = unreason. |
| 13 | NLG WOFFW Proxi unreasonable | 0 = not unreason. 1 = unreason. |
| 14 | MLG LH downlock Proxi unreasonable | 0 = not unreason. 1 = unreason. |
| 15 | MLG RH dowlock Proxi unreasonable | 0 = not unreason. 1 = unreason. |
| 16 | NLG downlock Proxi unreasonable | 0 = not unreason. 1 = unreason. |
| 17 | NLG uplock Proxi unreasonable | 0 = not unreason. 1 = unreason. |
| 18 | MLG RH uplock Proxi unreasonable | 0 = not unreason. 1 = unreason. |
| 19 | MLG LH uplock Proxi unreasonable | 0 = not unreason. 1 = unreason. |
| 20 | Spare |  |
| 21 |  |  |
| 22 |  |  |
| 23 |  |  |
| 24 |  |  |
| 25 |  |  |
| 26 |  |  |
| 27 |  |  |
| 28 |  |  |
| 29 | Spare |  |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

#### 2.14  Label TBD LGCU 1/2 Maintenance Word 9 (Electrical Fault of LG Prox)

| Label | 157（八进制） |  |
| --- | --- | --- |
| Name | LGCU 1/2 Maintenance Word 9 (Electrical Fault of LG Prox) |  |
| Description | LG sensor fault information; Transmitted by LGCU CC |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | MLG LH WOFFW Proxi Fault | 0 = no Fault 1 = Fault |
| 12 | MLG RH WOFFW Proxi Fault | 0 = no Fault 1 = Fault |
| 13 | NLG WOFFW Proxi Fault | 0 = no Fault 1 = Fault |
| 14 | MLG LH downlock Proxi Fault | 0 = no Fault 1 = Fault |
| 15 | MLG RH dowlock Proxi Fault | 0 = no Fault 1 = Fault |
| 16 | NLG downlock Proxi Fault | 0 = no Fault 1 = Fault |
| 17 | NLG uplock Proxi Fault | 0 = no Fault 1 = Fault |
| 18 | MLG RH uplock Proxi Fault | 0 = no Fault 1 = Fault |
| 19 | MLG LH uplock Proxi Fault | 0 = no Fault 1 = Fault |
| 20 | Spare |  |
| 21 | DO CONS MLG WOW Fault | 0 = no Fault 1 = Fault |
| 22 | DO MLG LH WOW Fault | 0 = no Fault 1 = Fault |
| 23 | DO MLG RH WOW Fault | 0 = no Fault 1 = Fault |
| 24 | DO CONS MLG UPLK Fault | 0 = no Fault 1 = Fault |
| 25 | DO CONS MLG NOT DNLK Fault | 0 = no Fault 1 = Fault |
| 26 | DC BUS Power Supply Low | 0 = not low 1 = low |
| 27 | Spare |  |
| 28 | AE Valve mechanical Fault | 0 = no Fault 1 = Fault |
| 29 | DC ESS BUS Power Supply Low | 0 = not low 1 = low |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |

#### 2.15  Label TBD LGCU 1/2 Maintenance Word 12 (LGCU internal faults)

| Label | 158（八进制） |  |
| --- | --- | --- |
| Name | LGCU 1/2 Maintenance Word 12 (LGCU internal faults) |  |
| Description | Transmitted by LGCU CC |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: A429_LGCU1_ConCh_Tx1 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 | MC ADC BIT Flt | 0 = no Fault 1 = Fault |
| 12 | CC ADC BIT Flt | 0 = no Fault 1 = Fault |
| 13 | DAC BIT Flt (LGCU1 only) | 0 = no Fault 1 = Fault |
| 14 | EnVlvDrvIct Flt (LGCU1 only) | 0 = no Fault 1 = Fault |
| 15 | MC EnVlvDrvEnCld Flt (LGCU1 only) | 0 = no Fault 1 = Fault |
| 16 | Spare |  |
| 17 | AevDoPos2Mon Flt (LGCU2 only) | 0 = no Fault 1 = Fault |
| 18 | MC SwInopMonCirTest Flt | 0 = no Fault 1 = Fault |
| 19 | CC SwInopMonCirTest Flt | 0 = no Fault 1 = Fault |
| 20 | PowIntEvalCirTest Flt | 0 = no Fault 1 = Fault |
| 21 | AevDoPos1Mon Flt | 0 = no Fault 1 = Fault |
| 22 | ExtDoMon Flt | 0 = no Fault 1 = Fault |
| 23 | CmdAsymMon Flt (LGCU1 only) | 0 = no Fault 1 = Fault |
| 24 | Internal Bus Flt | 0 = no Fault 1 = Fault |
| 25 | EEPROM CalibDataBIT Flt | 0 = no Fault 1 = Fault |
| 26 | MC TimingTest Flt | 0 = no Fault 1 = Fault |
| 27 | DrOpenDoMon Flt | 0 = no Fault 1 = Fault |
| 28 | DrCldDoMon Flt | 0 = no Fault 1 = Fault |
| 29 | RetDoMon Flt | 0 = no Fault 1 = Fault |
| 30-31 | Status Matrix | 31,30 Description 11: Normal Operation 01: Not used 10: Not used 00: Failure Warning |
| 32 | Parity - Odd |  |

Note: For Label 364:

Bit 13 DAC BIT Flt: LGCU1 Only

Bit 14 EnVlvDrvIct Flt: LGCU1 Only

Bit 15 MC EnVlvDrvIct Flt: LGCU1 Only

Bit 17 AevDoPos2Mon Flt: LGCU2 Only

Bit 23 CmdAsymMon Flt: LGCU1 Only

Bit 24 Internal Bus Flt: 控制通道和监控通道交互通讯

#### 2.16  Label TBD LGCU 2 Maintenance Word 13 (Raw Discrete Input Data)

| Label | 159（八进制） |  |
| --- | --- | --- |
| Name | LGCU 2 Maintenance Word 13 (Raw Discrete Input Data) |  |
| Description | Raw discrete input data from Maintenance Switch, Uplocks, Relays and LGCP Transmitted by LGCU CC |  |
| BIT | FUNCTION | BIT STATUS |
| 1-8 | Label (MSB at Bit 1, LSB at Bit 8) |  |
| 9-10 | SDI | 10,9 Description 00: Not used 01: Not used 10: A429_LGCU2_ConCh_Tx1 11: Not used |
| 11 |  |  |
| 12 |  |  |
| 13 | AEV2_28V_POS1_DISC | 0=false 1=true |
| 14 | AEV2_28V_POS2_DISC | 0=false 1=true |
| 15 | LGCL/AES_28V_EN | 0=false 1=true |
| 16 | Spare |  |
| 17 | RH_MLG_UPLOCK_GND_DISC | 0=false 1=true |
| 18 | LH_MLG_UPLOCK_GND_DISC | 0=false 1=true |
| 19 | NLG_UPLOCK_GND_DISC | 0=false 1=true |
| 20 |  |  |
| 21 |  |  |
| 22 |  |  |
| 23 | Spare |  |
| 24 | RH_MLG_UPLOCK_28V_DISC | 0=false 1=true |
| 25 | LH_MLG_UPLOCK_28V_DISC | 0=false 1=true |
| 26 | NLG_UPLOCK_28V_DISC | 0=false 1=true |
| 27 |  |  |
| 28 |  |  |
| 29 |  |  |
| 30-31 | Status Matrix | 31,30 Description 00: Normal Operation 01: Not used 10: Not used 11: Failure Warning |
| 32 | Parity - Odd |  |